WellPlanTM Report

04. КМГ-Бурение

Well Name: 146

Wellbore: 146

Design: Для МиГ S

Case: 215,9 мм

Date: October 6, 2023 at 12:12 AM

Created By:

- General Information

  - General Case Information

| Company | 04. КМГ-Бурение |  |  |
| --- | --- | --- | --- |
| Project | Восточный Mакат | Site | 146 ННС S-образ. |
| Well | 146 | Wellbore | 146 |
| Design | Для МиГ S | Case | 215,9 мм |
| Hole MD | 1069.00 m | Hole TVD | 1062.00 m |
| Air Gap | 0.00 m | Ground Elevation | m |
| Reference Point | @ | Well Type | Platform |

  - Active Fluid

    - Fluid Data

| Fluid | KC1 | Type | Mud |
| --- | --- | --- | --- |
| Mud Base Type | Water | Base Fluid | Water |
| Rheology Model | Bingham Plastic | Foamed | N |

    - Rheology Data

| Temperature (°C) | Pressure (atm) | Base Density (kg/m³) | Ref Fluid Properties | PV (Mulnf) (cp) | YP (Tau0) (lbf/100ft²) | Fann Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Speed (rpm) | Dial (°) |
| 45 | 1 | 1250 | Yes | 22 | 23 |  |  |

  - Hole Section

| Section Type | Section Depth (m) | Section Length (m) | Shoe Depth (m) | ID (mm) | Drift (mm) | Eff. Hole Diameter (mm) | Coefficient of Friction | Linear Capacity (L/m) | Volume Excess (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Casing | 350 | 350 | 350 | 226.7 | 222.73 |  | 0.25 | 40.36 |  |
| Open Hole | 1069 | 719 |  | 215.9 | 220.45 | 215.9 | 0.3 | 36.61 | 0 |

  - String Details

| Type | Length (m) | Depth (m) | Body |  | Stabilizer / Tool Joint |  |  |  | Weight | Material | Grade | Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OD (mm) | ID (mm) | Avg Joint Length (m) | Length (m) | OD (mm) | ID (mm) |  |  |  |  |

| Drill Pipe | 869.216 | 869.22 | 114.3 | 97.18 | 9.144 | 0.433 | 142.88 | 63.5 | 33.86 | CS_API 5D/7 | G | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Heavy Weight | 160 | 1029.22 | 127 | 76.2 | 9.144 | 1.219 | 165.1 | 76.2 | 73.13 | CS_1340 MOD | 1340 MOD |  |
| Jar | 9.07 | 1038.29 | 165 | 57 | 9.07 |  |  |  | 69.2 | SS_15-15LC | 15-15LC MOD (1) |  |
| Heavy Weight | 9.3 | 1047.59 | 127 | 76.2 | 9.144 | 1.219 | 165.1 | 76.2 | 73.13 | CS_1340 MOD | 1340 MOD |  |
| MWD | 10.4 | 1057.99 | 172 | 83 | 10.4 |  |  |  | 149.77 | SS_15-15LC | 15-15LC MOD (1) |  |
| Stabilizer | 1.524 | 1059.51 | 171.45 | 71.44 | 1.524 |  |  |  | 123.91 | CS_API 5D/7 | 4145H MOD |  |
| Sub | 0.96 | 1060.47 | 172 | 83 | 0.96 |  |  |  | 156.36 | SS_15-15LC | 15-15LC MOD (1) |  |
| Mud Motor | 8.2 | 1068.67 | 172 | 69.85 | 8.2 |  |  |  | 156.37 | CS_API 5D/7 | 4145H MOD |  |
| Bit | 0.33 | 1069 | 215.9 |  | 0.33 |  |  |  | 50 |  |  |  |

    - Grade in Use

| Grade | Minimum Yield Stress (psi) |
| --- | --- |
| 1340 MOD | 55,000 |
| 15-15LC MOD (1) | 110,000 |
| 4145H MOD | 110,000 |
| G | 105,000 |

    - String Nozzles

| Component | MD (m) | Port Open | Diverted Flow | Amount Diverted (%) | Nozzle (32nd") | TFA (in²) |
| --- | --- | --- | --- | --- | --- | --- |
| Polycrystalline Diamond Bit | 1,069 | NA | NA | NA | 8.0X14.0 | 1.203 |

    - Mud Motors

| Component | Length (m) | Steering tool |  |  | Kick pad |  |  | Pressure loss @ Flow rate ( @ ) | Lobe config | Eccentricity () | Rotor mass () | Rev. per Volume () |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bend angle (°) | Ref angle (°) | Offset (m) | Length (m) | OD (mm) | Offset (m) |  |  |  |  |  |

|  |  |  |  |  |  |  |  | @ @ @ @ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Wellpath - Calculation Method: Minimum Curvature

| MD (m) | INC (°) | AZ (°) | TVD (m) | DLS (°/30m) | AbsTort (°/30m) | RelTort (°/30m) | VSect (m) | NS (m) | EW (m) | Build (°/30m) | Walk (°/30m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 3.00 | 1.82 | 0.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 30.00 | 3.00 | 1.82 | 29.96 | 0.000 | 0.000 | 0.000 | -1.56 | 1.57 | 0.00 | 0.000 | 0.000 |
| 60.00 | 3.00 | 1.82 | 59.92 | 0.000 | 0.000 | 0.000 | -3.12 | 3.14 | 0.00 | 0.000 | 0.000 |
| 90.00 | 3.00 | 1.82 | 89.88 | 0.000 | 0.000 | 0.000 | -4.68 | 4.71 | 0.00 | 0.000 | 0.000 |
| 120.00 | 3.00 | 1.82 | 119.84 | 0.000 | 0.000 | 0.000 | -6.24 | 6.28 | 0.00 | 0.000 | 0.000 |
| 150.00 | 3.00 | 1.82 | 149.79 | 0.000 | 0.000 | 0.000 | -7.80 | 7.85 | 0.00 | 0.000 | 0.000 |
| 180.00 | 3.00 | 1.82 | 179.75 | 0.000 | 0.000 | 0.000 | -9.35 | 9.42 | 0.00 | 0.000 | 0.000 |
| 210.00 | 3.00 | 1.82 | 209.71 | 0.000 | 0.000 | 0.000 | -10.91 | 10.99 | 0.00 | 0.000 | 0.000 |
| 240.00 | 3.00 | 1.82 | 239.67 | 0.000 | 0.000 | 0.000 | -12.47 | 12.56 | 0.00 | 0.000 | 0.000 |
| 270.00 | 3.00 | 1.82 | 269.63 | 0.000 | 0.000 | 0.000 | -14.03 | 14.13 | 0.00 | 0.000 | 0.000 |
| 300.00 | 3.00 | 1.82 | 299.59 | 0.000 | 0.000 | 0.000 | -15.59 | 15.70 | 0.00 | 0.000 | 0.000 |
| 330.00 | 3.00 | 1.82 | 329.55 | 0.000 | 0.000 | 0.000 | -17.15 | 17.27 | 0.00 | 0.000 | 0.000 |
| 360.00 | 3.00 | 1.82 | 359.51 | 0.000 | 0.000 | 0.000 | -18.71 | 18.84 | 0.00 | 0.000 | 0.000 |
| 390.00 | 3.00 | 1.82 | 389.47 | 0.000 | 0.000 | 0.000 | -20.27 | 20.41 | 0.00 | 0.000 | 0.000 |
| 420.00 | 3.00 | 1.82 | 419.42 | 0.000 | 0.000 | 0.000 | -21.83 | 21.98 | 0.00 | 0.000 | 0.000 |
| 450.00 | 3.00 | 1.82 | 449.38 | 0.000 | 0.000 | 0.000 | -23.39 | 23.55 | 0.00 | 0.000 | 0.000 |
| 480.00 | 3.00 | 1.82 | 479.34 | 0.000 | 0.000 | 0.000 | -24.94 | 25.12 | 0.00 | 0.000 | 0.000 |
| 500.00 | 3.00 | 1.82 | 499.31 | 0.000 | 0.000 | 0.000 | -25.98 | 26.17 | 0.00 | 0.000 | 0.000 |
| 510.00 | 1.74 | 4.10 | 509.31 | 3.800 | 0.075 | 0.000 | -26.39 | 26.58 | 0.01 | -3.790 | 6.851 |
| 540.00 | 2.08 | 174.17 | 539.30 | 3.800 | 0.281 | 0.000 | -26.30 | 26.50 | 0.10 | 0.341 | 170.070 |
| 570.00 | 5.87 | 177.10 | 569.22 | 3.800 | 0.467 | 0.000 | -24.22 | 24.43 | 0.30 | 3.796 | 2.927 |
| 600.00 | 9.67 | 177.73 | 598.94 | 3.800 | 0.633 | 0.000 | -20.17 | 20.38 | 0.60 | 3.799 | 0.631 |
| 630.00 | 13.47 | 178.01 | 628.33 | 3.800 | 0.784 | 0.000 | -14.16 | 14.38 | 1.01 | 3.800 | 0.277 |
| 660.00 | 17.27 | 178.16 | 657.25 | 3.800 | 0.921 | 0.000 | -6.22 | 6.44 | 1.53 | 3.800 | 0.157 |
| 690.00 | 21.07 | 178.27 | 685.58 | 3.800 | 1.046 | 0.000 | 3.62 | -3.39 | 2.15 | 3.800 | 0.102 |
| 720.93 | 24.99 | 178.34 | 714.04 | 3.800 | 1.165 | 0.000 | 15.70 | -15.46 | 2.89 | 3.800 | 0.071 |
| 750.00 | 24.99 | 177.97 | 740.39 | 0.163 | 1.126 | 0.000 | 27.96 | -27.72 | 3.67 | -0.001 | -0.386 |
| 780.00 | 24.99 | 177.58 | 767.58 | 0.163 | 1.089 | 0.000 | 40.62 | -40.36 | 4.57 | 0.000 | -0.386 |
| 795.75 | 24.99 | 177.38 | 781.86 | 0.163 | 1.070 | 0.000 | 47.27 | -47.00 | 5.07 | 0.001 | -0.386 |
| 810.00 | 23.39 | 177.23 | 794.85 | 3.363 | 1.111 | 0.000 | 53.10 | -52.82 | 5.53 | -3.360 | -0.316 |
| 840.00 | 20.03 | 176.84 | 822.72 | 3.363 | 1.191 | 0.000 | 64.19 | -63.87 | 6.45 | -3.360 | -0.389 |
| 870.00 | 16.68 | 176.30 | 851.19 | 3.363 | 1.266 | 0.000 | 73.63 | -73.28 | 7.31 | -3.358 | -0.539 |
| 900.00 | 13.32 | 175.50 | 880.16 | 3.363 | 1.336 | 0.000 | 81.39 | -81.00 | 8.11 | -3.356 | -0.801 |
| 930.00 | 9.97 | 174.17 | 909.54 | 3.363 | 1.401 | 0.000 | 87.45 | -87.01 | 8.83 | -3.352 | -1.328 |
| 960.00 | 6.63 | 171.52 | 939.22 | 3.363 | 1.463 | 0.000 | 91.77 | -91.29 | 9.49 | -3.342 | -2.652 |
| 990.00 | 3.33 | 163.59 | 969.11 | 3.363 | 1.520 | 0.000 | 94.35 | -93.82 | 10.07 | -3.299 | -7.931 |
| 993.04 | 3.00 | 161.82 | 972.15 | 3.363 | 1.526 | 0.000 | 94.52 | -93.98 | 10.13 | -3.222 | -17.445 |
| 1020.00 | 3.00 | 161.82 | 999.06 | 0.000 | 1.485 | 0.000 | 95.89 | -95.31 | 10.61 | 0.000 | 0.000 |
| 1050.00 | 3.00 | 161.82 | 1029.02 | 0.000 | 1.443 | 0.000 | 97.42 | -96.78 | 11.15 | 0.000 | 0.000 |
| 1080.00 | 3.00 | 161.82 | 1058.98 | 0.000 | 1.403 | 0.000 | 98.95 | -98.26 | 11.68 | 0.000 | 0.000 |
| 1083.02 | 3.00 | 161.82 | 1062.00 | 0.000 | 1.399 | 0.000 | 99.10 | -98.41 | 11.74 | 0.000 | 0.000 |

  - Geothermal Gradient Data

| Ambient Temperature | 27.000 °C | Mudline Temperature | °C |
| --- | --- | --- | --- |
| Temperature @ Depth | 45.000 °C @ 1062.00 m | Gradient | 1.69 °C/100m |

- Schematics

| Well: | 146 | Wellbore: | 146 | Case: | 215,9 мм | String Name: | секция 215,9мм |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Torque & Drag Setup Data

  - Settings

| Measured Depth of Bit | 1069.00 m | Bending Stress Magnification | Yes |
| --- | --- | --- | --- |
| Block Weight | 15.00 tonne | Stiff String Analysis | No |
| Enable Sheave Friction Correction | No | Viscous Torque and Drag | No |
| Pump Rate | 30.000 L/sec | Contact Force Normalization Length | 9.30 m |
| Mechanical Efficiency (Single Sheave) | 97.00 | Lines Strung | 12 |
|  |  | Side Force | 0.00 kgf |
| Offset from Wellhead | m | Angle at Wellhead | ° |
| Buckling limit factor | 1 |  |  |

  - Run Parameters

| Start MD | 500.00 m | End MD | 1069.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Normal Analysis Operational Parameters

| Drilling | WOB/Overpull (tonne) | Torque at Bit (kN-m) | Include Pump Rate |
| --- | --- | --- | --- |
| Rotating On Bottom | 8.00 | 4.1650 | NA |
| Slide Drilling | 5.00 | 0.0000 | NA |
| Backreaming | NA | NA | NA |
| Rotating Off Bottom |  |  | NA |
| Tripping | Speed (m/min) | RPM (rpm) | Include Pump Rate |
| Tripping In | 10.00 | 0 | NA |
| Tripping Out | 10.00 | 0 | NA |

  - Friction Factors

| Section Type | Coefficient of Friction |
| --- | --- |
| Open Hole | 0.30 |
| Casing | 0.25 |

  - String Fill Up

| Use String Fill Up (Tripping In only) | No | Period | m |
| --- | --- | --- | --- |

- Torque and Drag Results

  - Mechanical Limitations

| Overpull Margin during a Tripping Out operation | 54.96 tonne | using | 80.00% of yield |
| --- | --- | --- | --- |
| Minimum Weight on Bit to Sinusoidal Buckle during a rotating on bottom operation | 13.82 tonne | at | 1047.59 m |
| Minimum Weight on Bit to Helical Buckle during a rotating on bottom operation | 18.42 tonne | at | 1047.59 m |
| Pick-Up Drag | 5.84 tonne |  |  |
| Slack-Off Drag | 4.59 tonne |  |  |
| Block Rating (Hoisting System) | 200.00 tonne |  |  |
| Torque Rating (Rotating Equipment) | kN-m |  |  |

  - Load Summary

| Load Condition | Stress Failure |  |  | Buckling Limits |  |  | Torque Failure | Torque at the Rotary Table (kN-m) | Total Windup with Bit Torque (revs) | Total Windup without Bit Torque (revs) | Measured Weight (tonne) | Total Stretch (m) | Axial Stress = 0 |  | Neutral Point Distance from surface (m) | Neutral Point Distance from Bit (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fatigue | 90% Yield | 100% Yield | Sinusoidal | Helical | Lockup |  |  |  |  |  |  | Distance from Surface (m) | Distance from Bit (m) |  |  |
| Спуск |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 49.10 | 0.42 | 909.18 | 159.82 | 1069.00 | 0.00 |
| Подъём |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 59.53 | 0.54 | 927.83 | 141.17 | 1069.00 | 0.00 |
| Бурение ротором |  |  |  |  |  |  |  | 6.3784 | 1.3 | 0.4 | 45.69 | 0.35 | 869.22 | 199.78 | 959.13 | 109.87 |
| Бурение ГЗД |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 45.26 | 0.36 | 869.22 | 199.78 | 1006.94 | 62.06 |
| Вращение над забоем |  |  |  |  |  |  |  | 3.7484 | 0.6 | 0.6 | 53.69 | 0.47 | 919.54 | 149.46 | 1069.00 | 0.00 |

- Torque and Drag Plots

- Эффективное натяжение

- Вес на крюке

- Момент

- Мин. вес на долоте

- Hydraulics Setup Data

  - Calculation Engine

| Model Used | WellPlan |  |
| --- | --- | --- |

  - Cuttings Loading Calculation Option

| Rate of Penetration | 18.00 m/hr | Rotary Speed | 40 rpm |
| --- | --- | --- | --- |
| Cuttings Diameter | 3.18 mm | Cuttings Density | 2.500 sg |
| Bed Porosity | 36.00 % | MD Calculation Interval | 30.48 m |

  - Surface Equipment Information

| Pressure Loss Calculation | Specify Pressure loss | Maximum Working Pressure | 200.0000 atm |
| --- | --- | --- | --- |
| Equipment Mode | NA | Surface Pressure Loss | 6.8046 atm |
| Equipment Type | NA |  |  |

  - Pump Pressure Information

| Maximum Surface Pressure | 200.0000 atm | Pump Rate | 30.000 L/sec |
| --- | --- | --- | --- |
| Maximum Pump Power | hp | Maximum Allowable Pump Rate | L/sec |
| Use Roughness | N |  |  |
| Pipe Roughness | NA | Annulus Roughness | NA |
| Booster Pump |  | Injection Depth |  |
| Injection Temperature |  | Injection Rate |  |
| Include Tool Joint Pressure Losses | N |  |  |
| Include Back Pressure |  | Back Pressure | 0.0000 atm |
| Sea Floor Returns | N | Sea Water Density | NA |

  - Run Parameters

| Start MD | 500.00 m | End MD | 1069.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Flow Rate (Q= 30.000 L/sec)

    - Bit Parameters

| Pump Rate | 30.000 L/sec | Stand Pipe Pressure | 138.7624 atm |
| --- | --- | --- | --- |
| Bit Pressure Loss | 10.2177 atm | Percent Power at Bit | 7.36 % |
| Bit Hydraulic Power / Area (HSI) | 0.7 hp/in² | Bit Nozzle Velocity | 38.66 m/s |
| Bit Hydraulic Power | 41.65 hp | Bit Impact Force | 147.85 kgf |
| Surface Equip. Pressure Loss | 6.8046 atm | Total Bit Flow Area | 1.203 in² |

  - Gel Strength

| 0 Second | 5.000 lbf/100ft² | 10 Second | 9.000 lbf/100ft² |
| --- | --- | --- | --- |
| 10 Minute | 18.000 lbf/100ft² | 30 Minute | 25.000 lbf/100ft² |
| Maximum | lbf/100ft² |  |  |

  - Mud Temperature Information

| Include Mud Temperature Effects | N | Circulation Time | NA |
| --- | --- | --- | --- |

- Hydraulics Plots

- Потери давления на долоте

- Мин. расход по глубине

- Высота шламовой подушки по глубине

- Потери мощности компонента

- Потери давления компонента