WellPlanTM Report

04. КМГ-Бурение

Well Name: 81

Wellbore: 81

Design: Design #1

Case: 2159

Date: October 6, 2023 at 12:35 AM

Created By:

- General Information

  - General Case Information

| Company | 04. КМГ-Бурение |  |  |
| --- | --- | --- | --- |
| Project | Гран | Site | 81 |
| Well | 81 | Wellbore | 81 |
| Design | Design #1 | Case | 2159 |
| Hole MD | 566.00 m | Hole TVD | 449.37 m |
| Air Gap | 0.00 m | Ground Elevation | -17.90 m |
| Reference Point | WELL (copy) @ -17.9m | Well Type | Platform |

  - Active Fluid

    - Fluid Data

| Fluid | KCl | Type | Mud |
| --- | --- | --- | --- |
| Mud Base Type | Water | Base Fluid | Water |
| Rheology Model | Bingham Plastic | Foamed | N |

    - Rheology Data

| Temperature (°C) | Pressure (atm) | Base Density (kg/m³) | Ref Fluid Properties | PV (Mulnf) (cp) | YP (Tau0) (lbf/100ft²) | Fann Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Speed (rpm) | Dial (°) |
| 33 | 1 | 1200 | Yes | 18 | 20 |  |  |

  - Hole Section

| Section Type | Section Depth (m) | Section Length (m) | Shoe Depth (m) | ID (mm) | Drift (mm) | Eff. Hole Diameter (mm) | Coefficient of Friction | Linear Capacity (L/m) | Volume Excess (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Casing | 170 | 170 | 170 | 224.41 | 222.25 | 295.3 | 0.25 | 39.55 |  |
| Open Hole | 566 | 396 |  | 215.9 | 222.25 | 215.9 | 0.3 | 36.61 | 0 |

  - String Details

| Type | Length (m) | Depth (m) | Body |  | Stabilizer / Tool Joint |  |  |  | Weight | Material | Grade | Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OD (mm) | ID (mm) | Avg Joint Length (m) | Length (m) | OD (mm) | ID (mm) |  |  |  |  |

| Drill Pipe | 294.522 | 294.52 | 127 | 108.61 | 9.144 | 0.433 | 152.4 | 82.55 | 32.62 | CS_API 5D/7 | G | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Heavy Weight | 60 | 354.52 | 127 | 76.2 | 9.14 | 1.219 | 165.1 | 76.2 | 73.13 | CS_1340 MOD | 1340 MOD |  |
| Jar | 10.058 | 364.58 | 165.1 | 69.85 | 10.058 |  |  |  | 136.6 | CS_API 5D/7 | 4145H MOD |  |
| Heavy Weight | 40 | 404.58 | 127 | 76.2 | 9.14 | 1.219 | 165.1 | 76.2 | 73.13 | CS_1340 MOD | 1340 MOD |  |
| Drill Pipe | 140 | 544.58 | 127 | 108.61 | 9.144 | 0.433 | 152.4 | 82.55 | 32.62 | CS_API 5D/7 | G | 2 |
| MWD | 10.4 | 554.98 | 172 | 83 | 10.4 |  |  |  | 149.77 | SS_15-15LC | 15-15LC MOD (1) |  |
| Sub | 0.91 | 555.89 | 170.69 | 60.96 | 0.91 |  |  |  | 156.36 | SS_15-15LC | 15-15LC MOD (1) |  |
| Mud Motor | 9.71 | 565.6 | 171.45 | 76.2 | 9.71 |  |  |  | 103.53 | CS_API 5D/7 | 4145H MOD |  |
| Bit | 0.4 | 566 | 215.9 |  | 0.4 |  |  |  | 100 |  |  |  |

    - Grade in Use

| Grade | Minimum Yield Stress (psi) |
| --- | --- |
| 1340 MOD | 55,000 |
| 15-15LC MOD (1) | 110,000 |
| 4145H MOD | 110,000 |
| G | 105,000 |

    - String Nozzles

| Component | MD (m) | Port Open | Diverted Flow | Amount Diverted (%) | Nozzle (32nd") | TFA (in²) |
| --- | --- | --- | --- | --- | --- | --- |
| Polycrystalline Diamond Bit | 566 | NA | NA | NA | 7.0X14.0 | 1.052 |

    - Mud Motors

| Component | Length (m) | Steering tool |  |  | Kick pad |  |  | Pressure loss @ Flow rate ( @ ) | Lobe config | Eccentricity () | Rotor mass () | Rev. per Volume () |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bend angle (°) | Ref angle (°) | Offset (m) | Length (m) | OD (mm) | Offset (m) |  |  |  |  |  |

|  |  |  |  |  |  |  |  | @ @ @ @ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Wellpath - Calculation Method: Minimum Curvature

| MD (m) | INC (°) | AZ (°) | TVD (m) | DLS (°/30m) | AbsTort (°/30m) | RelTort (°/30m) | VSect (m) | NS (m) | EW (m) | Build (°/30m) | Walk (°/30m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 | 359.98 | 0.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 30.00 | 0.00 | 359.98 | 30.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 60.00 | 0.00 | 359.98 | 60.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 90.00 | 0.00 | 359.98 | 90.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 120.00 | 0.00 | 359.98 | 120.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 150.00 | 0.00 | 359.98 | 150.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 180.00 | 0.00 | 359.98 | 180.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 200.00 | 0.00 | 359.98 | 200.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 210.00 | 2.50 | 322.98 | 210.00 | 7.500 | 0.357 | 0.000 | 0.22 | 0.17 | -0.13 | 7.500 | 0.000 |
| 225.00 | 6.25 | 322.98 | 224.95 | 7.500 | 0.833 | 0.000 | 1.36 | 1.09 | -0.82 | 7.500 | 0.000 |
| 240.00 | 10.00 | 322.98 | 239.80 | 7.500 | 1.250 | 0.000 | 3.47 | 2.78 | -2.10 | 7.500 | 0.000 |
| 255.00 | 13.75 | 322.98 | 254.47 | 7.500 | 1.618 | 0.000 | 6.55 | 5.25 | -3.95 | 7.500 | 0.000 |
| 270.00 | 17.50 | 322.98 | 268.92 | 7.500 | 1.944 | 0.000 | 10.58 | 8.47 | -6.38 | 7.500 | 0.000 |
| 285.00 | 21.25 | 322.98 | 283.06 | 7.500 | 2.237 | 0.000 | 15.55 | 12.44 | -9.38 | 7.500 | 0.000 |
| 300.00 | 25.00 | 322.98 | 296.86 | 7.500 | 2.500 | 0.000 | 21.42 | 17.15 | -12.92 | 7.500 | 0.000 |
| 315.00 | 28.75 | 322.98 | 310.23 | 7.500 | 2.738 | 0.000 | 28.19 | 22.56 | -17.00 | 7.500 | 0.000 |
| 330.00 | 32.50 | 322.98 | 323.14 | 7.500 | 2.955 | 0.000 | 35.81 | 28.66 | -21.60 | 7.500 | 0.000 |
| 345.00 | 36.25 | 322.98 | 335.52 | 7.500 | 3.152 | 0.000 | 44.26 | 35.43 | -26.70 | 7.500 | 0.000 |
| 360.00 | 40.00 | 322.98 | 347.32 | 7.500 | 3.333 | 0.000 | 53.49 | 42.82 | -32.27 | 7.500 | 0.000 |
| 375.00 | 43.75 | 322.98 | 358.48 | 7.500 | 3.500 | 0.000 | 63.48 | 50.82 | -38.29 | 7.500 | 0.000 |
| 380.00 | 45.00 | 322.98 | 362.06 | 7.500 | 3.553 | 0.000 | 66.97 | 53.61 | -40.40 | 7.500 | 0.000 |
| 390.00 | 45.00 | 322.98 | 369.13 | 0.000 | 3.462 | 0.000 | 74.02 | 59.26 | -44.65 | 0.000 | 0.000 |
| 405.00 | 48.71 | 322.98 | 379.38 | 7.428 | 3.608 | 0.000 | 84.94 | 68.00 | -51.24 | 7.428 | 0.000 |
| 420.00 | 52.43 | 322.98 | 388.91 | 7.428 | 3.745 | 0.000 | 96.50 | 77.25 | -58.21 | 7.428 | 0.000 |
| 435.00 | 56.14 | 322.98 | 397.66 | 7.428 | 3.872 | 0.000 | 108.64 | 86.97 | -65.54 | 7.428 | 0.000 |
| 450.00 | 59.86 | 322.98 | 405.61 | 7.428 | 3.990 | 0.000 | 121.33 | 97.13 | -73.19 | 7.428 | 0.000 |
| 465.00 | 63.57 | 322.98 | 412.72 | 7.428 | 4.101 | 0.000 | 134.51 | 107.68 | -81.14 | 7.428 | 0.000 |
| 480.00 | 67.28 | 322.98 | 418.95 | 7.428 | 4.205 | 0.000 | 148.12 | 118.57 | -89.35 | 7.428 | 0.000 |
| 495.00 | 71.00 | 322.98 | 424.29 | 7.428 | 4.303 | 0.000 | 162.10 | 129.76 | -97.78 | 7.428 | 0.000 |
| 510.00 | 74.71 | 322.98 | 428.72 | 7.428 | 4.395 | 0.000 | 176.39 | 141.21 | -106.41 | 7.428 | 0.000 |
| 525.00 | 78.43 | 322.98 | 432.20 | 7.428 | 4.481 | 0.000 | 190.95 | 152.86 | -115.19 | 7.428 | 0.000 |
| 540.00 | 82.14 | 322.98 | 434.73 | 7.428 | 4.563 | 0.000 | 205.69 | 164.66 | -124.08 | 7.428 | 0.000 |
| 555.00 | 85.85 | 322.98 | 436.30 | 7.428 | 4.641 | 0.000 | 220.57 | 176.57 | -133.06 | 7.428 | 0.000 |
| 566.49 | 88.70 | 322.98 | 436.85 | 7.428 | 4.697 | 0.000 | 232.03 | 185.74 | -139.97 | 7.428 | 0.000 |
| 570.00 | 88.70 | 323.33 | 436.92 | 3.000 | 4.687 | 0.000 | 235.53 | 188.55 | -142.07 | -0.003 | 3.001 |
| 600.00 | 88.70 | 326.33 | 437.61 | 3.000 | 4.603 | 0.000 | 265.49 | 213.07 | -159.33 | -0.001 | 3.001 |
| 625.48 | 88.70 | 328.88 | 438.18 | 3.000 | 4.537 | 0.000 | 290.96 | 234.58 | -172.97 | 0.002 | 3.001 |
| 630.00 | 88.70 | 328.88 | 438.29 | 0.000 | 4.505 | 0.000 | 295.48 | 238.45 | -175.31 | 0.000 | 0.000 |
| 660.00 | 88.70 | 328.88 | 438.97 | 0.000 | 4.300 | 0.000 | 325.45 | 264.13 | -190.80 | 0.000 | 0.000 |
| 690.00 | 88.70 | 328.88 | 439.65 | 0.000 | 4.113 | 0.000 | 355.43 | 289.81 | -206.29 | 0.000 | 0.000 |
| 720.00 | 88.70 | 328.88 | 440.33 | 0.000 | 3.942 | 0.000 | 385.40 | 315.49 | -221.78 | 0.000 | 0.000 |
| 750.00 | 88.70 | 328.88 | 441.01 | 0.000 | 3.784 | 0.000 | 415.38 | 341.17 | -237.27 | 0.000 | 0.000 |
| 780.00 | 88.70 | 328.88 | 441.69 | 0.000 | 3.638 | 0.000 | 445.35 | 366.86 | -252.77 | 0.000 | 0.000 |
| 810.00 | 88.70 | 328.88 | 442.37 | 0.000 | 3.504 | 0.000 | 475.33 | 392.54 | -268.26 | 0.000 | 0.000 |
| 840.00 | 88.70 | 328.88 | 443.05 | 0.000 | 3.379 | 0.000 | 505.30 | 418.22 | -283.75 | 0.000 | 0.000 |
| 870.00 | 88.70 | 328.88 | 443.73 | 0.000 | 3.262 | 0.000 | 535.28 | 443.90 | -299.24 | 0.000 | 0.000 |
| 900.00 | 88.70 | 328.88 | 444.41 | 0.000 | 3.153 | 0.000 | 565.25 | 469.58 | -314.73 | 0.000 | 0.000 |
| 930.00 | 88.70 | 328.88 | 445.09 | 0.000 | 3.052 | 0.000 | 595.23 | 495.26 | -330.23 | 0.000 | 0.000 |
| 960.00 | 88.70 | 328.88 | 445.77 | 0.000 | 2.956 | 0.000 | 625.20 | 520.94 | -345.72 | 0.000 | 0.000 |
| 990.00 | 88.70 | 328.88 | 446.45 | 0.000 | 2.867 | 0.000 | 655.17 | 546.63 | -361.21 | 0.000 | 0.000 |
| 1020.00 | 88.70 | 328.88 | 447.14 | 0.000 | 2.782 | 0.000 | 685.15 | 572.31 | -376.70 | 0.000 | 0.000 |
| 1050.00 | 88.70 | 328.88 | 447.82 | 0.000 | 2.703 | 0.000 | 715.12 | 597.99 | -392.19 | 0.000 | 0.000 |
| 1080.00 | 88.70 | 328.88 | 448.50 | 0.000 | 2.628 | 0.000 | 745.10 | 623.67 | -407.69 | 0.000 | 0.000 |
| 1110.00 | 88.70 | 328.88 | 449.18 | 0.000 | 2.557 | 0.000 | 775.07 | 649.35 | -423.18 | 0.000 | 0.000 |
| 1118.63 | 88.70 | 328.88 | 449.37 | 0.000 | 2.537 | 0.000 | 783.69 | 656.74 | -427.63 | 0.000 | 0.000 |

  - Geothermal Gradient Data

| Ambient Temperature | 27.000 °C | Mudline Temperature | °C |
| --- | --- | --- | --- |
| Temperature @ Depth | 33.000 °C @ 449.37 m | Gradient | 1.34 °C/100m |

- Schematics

| Well: | 81 | Wellbore: | 81 | Case: | 2159 | String Name: | 215,9 мм |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Torque & Drag Setup Data

  - Settings

| Measured Depth of Bit | 566.00 m | Bending Stress Magnification | Yes |
| --- | --- | --- | --- |
| Block Weight | 17.00 tonne | Stiff String Analysis | No |
| Enable Sheave Friction Correction | No | Viscous Torque and Drag | No |
| Pump Rate | 30.000 L/sec | Contact Force Normalization Length | 9.30 m |
| Mechanical Efficiency (Single Sheave) | 97.00 | Lines Strung | 12 |
|  |  | Side Force | 0.00 kgf |
| Offset from Wellhead | m | Angle at Wellhead | ° |
| Buckling limit factor | 1 |  |  |

  - Run Parameters

| Start MD | 200.00 m | End MD | 566.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Normal Analysis Operational Parameters

| Drilling | WOB/Overpull (tonne) | Torque at Bit (kN-m) | Include Pump Rate |
| --- | --- | --- | --- |
| Rotating On Bottom | 6.00 | 4.1670 | NA |
| Slide Drilling | 3.00 | 0.0000 | NA |
| Backreaming | NA | NA | NA |
| Rotating Off Bottom |  |  | NA |
| Tripping | Speed (m/min) | RPM (rpm) | Include Pump Rate |
| Tripping In | 10.00 | 0 | NA |
| Tripping Out | 10.00 | 0 | NA |

  - Friction Factors

| Section Type | Coefficient of Friction |
| --- | --- |
| Casing | 0.25 |
| Open Hole | 0.30 |

  - String Fill Up

| Use String Fill Up (Tripping In only) | No | Period | m |
| --- | --- | --- | --- |

- Torque and Drag Results

  - Mechanical Limitations

| Overpull Margin during a Tripping Out operation | 75.74 tonne | using | 80.00% of yield |
| --- | --- | --- | --- |
| Minimum Weight on Bit to Sinusoidal Buckle during a rotating on bottom operation | 11.30 tonne | at | 190.57 m |
| Minimum Weight on Bit to Helical Buckle during a rotating on bottom operation | 11.67 tonne | at | 190.57 m |
| Pick-Up Drag | 3.28 tonne |  |  |
| Slack-Off Drag | 3.42 tonne |  |  |
| Block Rating (Hoisting System) | 225.00 tonne |  |  |
| Torque Rating (Rotating Equipment) | kN-m |  |  |

  - Load Summary

| Load Condition | Stress Failure |  |  | Buckling Limits |  |  | Torque Failure | Torque at the Rotary Table (kN-m) | Total Windup with Bit Torque (revs) | Total Windup without Bit Torque (revs) | Measured Weight (tonne) | Total Stretch (m) | Axial Stress = 0 |  | Neutral Point Distance from surface (m) | Neutral Point Distance from Bit (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fatigue | 90% Yield | 100% Yield | Sinusoidal | Helical | Lockup |  |  |  |  |  |  | Distance from Surface (m) | Distance from Bit (m) |  |  |
| Спуск |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 29.30 | 0.24 | 317.54 | 248.46 | 566.00 | 0.00 |
| Подъём |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 36.00 | 0.27 | 469.20 | 96.80 | 566.00 | 0.00 |
| Бурение ротором |  |  |  |  |  |  |  | 7.7435 | 0.6 | 0.2 | 26.73 | 0.22 | 294.52 | 271.48 | 322.65 | 243.35 |
| Бурение ГЗД |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 25.48 | 0.21 | 263.81 | 302.19 | 303.84 | 262.16 |
| Вращение над забоем |  |  |  |  |  |  |  | 2.5544 | 0.2 | 0.2 | 32.73 | 0.25 | 354.52 | 211.48 | 566.00 | 0.00 |

- Torque and Drag Plots

- Эффективное натяжение

- Вес на крюке

- Момент

- Мин. вес на долоте

- Hydraulics Setup Data

  - Calculation Engine

| Model Used | WellPlan |  |
| --- | --- | --- |

  - Cuttings Loading Calculation Option

| Rate of Penetration | 8.00 m/hr | Rotary Speed | 40 rpm |
| --- | --- | --- | --- |
| Cuttings Diameter | 3.18 mm | Cuttings Density | 2.500 sg |
| Bed Porosity | 36.00 % | MD Calculation Interval | 30.48 m |

  - Surface Equipment Information

| Pressure Loss Calculation | Specify Pressure loss | Maximum Working Pressure | 270.0000 atm |
| --- | --- | --- | --- |
| Equipment Mode | NA | Surface Pressure Loss | 6.8046 atm |
| Equipment Type | NA |  |  |

  - Pump Pressure Information

| Maximum Surface Pressure | 350.0000 atm | Pump Rate | 30.000 L/sec |
| --- | --- | --- | --- |
| Maximum Pump Power | hp | Maximum Allowable Pump Rate | L/sec |
| Use Roughness | N |  |  |
| Pipe Roughness | NA | Annulus Roughness | NA |
| Booster Pump |  | Injection Depth |  |
| Injection Temperature |  | Injection Rate |  |
| Include Tool Joint Pressure Losses | N |  |  |
| Include Back Pressure |  | Back Pressure | 0.0000 atm |
| Sea Floor Returns | N | Sea Water Density | NA |

  - Run Parameters

| Start MD | 200.00 m | End MD | 566.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Flow Rate (Q= 30.000 L/sec)

    - Bit Parameters

| Pump Rate | 30.000 L/sec | Stand Pipe Pressure | 89.4989 atm |
| --- | --- | --- | --- |
| Bit Pressure Loss | 12.8118 atm | Percent Power at Bit | 14.32 % |
| Bit Hydraulic Power / Area (HSI) | 0.9 hp/in² | Bit Nozzle Velocity | 44.19 m/s |
| Bit Hydraulic Power | 52.23 hp | Bit Impact Force | 162.22 kgf |
| Surface Equip. Pressure Loss | 6.8046 atm | Total Bit Flow Area | 1.052 in² |

  - Gel Strength

| 0 Second | 5.000 lbf/100ft² | 10 Second | 9.000 lbf/100ft² |
| --- | --- | --- | --- |
| 10 Minute | 18.000 lbf/100ft² | 30 Minute | 25.000 lbf/100ft² |
| Maximum | lbf/100ft² |  |  |

  - Mud Temperature Information

| Include Mud Temperature Effects | N | Circulation Time | NA |
| --- | --- | --- | --- |

- Hydraulics Plots

- Потери давления на долоте

- Мин. расход по глубине

- Высота шламовой подушки по глубине

- Потери мощности компонента

- Потери давления компонента