WellPlanTM Report

04. КМГ-Бурение

Well Name: 2737

Wellbore: 2737

Design: План с полкой рев 02

Case: 2159

Date: October 5, 2023 at 5:35 PM

Created By:

- General Information

  - General Case Information

| Company | 04. КМГ-Бурение |  |  |
| --- | --- | --- | --- |
| Project | Молдабек | Site | 2737 new |
| Well | 2737 | Wellbore | 2737 |
| Design | План с полкой рев 02 | Case | 2159 |
| Hole MD | 404.00 m | Hole TVD | 355.85 m |
| Air Gap | 0.00 m | Ground Elevation | 0.00 m |
| Reference Point | WELL @ 0m | Well Type | Onshore |

  - Active Fluid

    - Fluid Data

| Fluid | KC1 | Type | Mud |
| --- | --- | --- | --- |
| Mud Base Type | Water | Base Fluid | Water |
| Rheology Model | Bingham Plastic | Foamed | N |

    - Rheology Data

| Temperature (°C) | Pressure (atm) | Base Density (kg/m³) | Ref Fluid Properties | PV (Mulnf) (cp) | YP (Tau0) (lbf/100ft²) | Fann Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Speed (rpm) | Dial (°) |
| 30 | 1 | 1170 | Yes | 14 | 18 |  |  |

  - Hole Section

| Section Type | Section Depth (m) | Section Length (m) | Shoe Depth (m) | ID (mm) | Drift (mm) | Eff. Hole Diameter (mm) | Coefficient of Friction | Linear Capacity (L/m) | Volume Excess (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Casing | 160 | 160 | 160 | 224.41 | 222.25 |  | 0.25 | 39.55 |  |
| Open Hole | 404 | 244 |  | 215.9 | 222.25 | 215.9 | 0.36 | 36.61 | 0 |

  - String Details

| Type | Length (m) | Depth (m) | Body |  | Stabilizer / Tool Joint |  |  |  | Weight | Material | Grade | Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OD (mm) | ID (mm) | Avg Joint Length (m) | Length (m) | OD (mm) | ID (mm) |  |  |  |  |

| Drill Pipe | 346.566 | 346.57 | 127 | 108.61 | 9.144 | 0.433 | 152.4 | 82.55 | 32.62 | CS_API 5D/7 | G | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Heavy Weight | 36 | 382.57 | 127 | 76.2 | 9.14 | 1.219 | 165.1 | 76.2 | 73.13 | CS_1340 MOD | 1340 MOD |  |
| MWD | 10.4 | 392.97 | 172 | 83 | 10.4 |  |  |  | 149.77 | SS_15-15LC | 15-15LC MOD (1) |  |
| Sub | 0.91 | 393.88 | 170.69 | 60.96 | 0.91 |  |  |  | 156.36 | SS_15-15LC | 15-15LC MOD (1) |  |
| Mud Motor | 9.71 | 403.59 | 171.45 | 76.2 | 9.71 |  |  |  | 103.53 | CS_API 5D/7 | 4145H MOD |  |
| Bit | 0.414 | 404 | 215.9 |  | 0.414 |  |  |  | 150 |  |  |  |

    - Grade in Use

| Grade | Minimum Yield Stress (psi) |
| --- | --- |
| 1340 MOD | 55,000 |
| 15-15LC MOD (1) | 110,000 |
| 4145H MOD | 110,000 |
| G | 105,000 |

    - String Nozzles

| Component | MD (m) | Port Open | Diverted Flow | Amount Diverted (%) | Nozzle (32nd") | TFA (in²) |
| --- | --- | --- | --- | --- | --- | --- |
| Polycrystalline Diamond Bit | 404 | NA | NA | NA | 5.0X14.0 | 0.752 |

    - Mud Motors

| Component | Length (m) | Steering tool |  |  | Kick pad |  |  | Pressure loss @ Flow rate ( @ ) | Lobe config | Eccentricity () | Rotor mass () | Rev. per Volume () |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bend angle (°) | Ref angle (°) | Offset (m) | Length (m) | OD (mm) | Offset (m) |  |  |  |  |  |

|  |  |  |  |  |  |  |  | @ @ @ @ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Tortuosity (Random Inc and Az)

| Tortuosity Period | 30.00 m |
| --- | --- |
| Interpolation Interval | 30.00 m |

| Measured Depth Top (m) | Magnitude (°) |
| --- | --- |
| 0.00 | 0.50 |
| 160.00 | 1.00 |
| 465.00 | 1.50 |

  - Wellpath - Calculation Method: Minimum Curvature

| MD (m) | INC (°) | AZ (°) | TVD (m) | DLS (°/30m) | AbsTort (°/30m) | RelTort (°/30m) | VSect (m) | NS (m) | EW (m) | Build (°/30m) | Walk (°/30m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 | 357.88 | 0.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 30.00 | 0.00 | 357.88 | 30.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 40.00 | 0.00 | 357.88 | 40.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 60.00 | 1.67 | 357.88 | 60.00 | 2.500 | 0.833 | 0.000 | 0.29 | 0.29 | 0.00 | 2.500 | 0.000 |
| 90.00 | 4.17 | 357.88 | 89.96 | 2.500 | 1.389 | 0.000 | 1.82 | 1.82 | 0.00 | 2.500 | 0.000 |
| 120.00 | 6.67 | 357.88 | 119.82 | 2.500 | 1.667 | 0.000 | 4.65 | 4.65 | 0.00 | 2.500 | 0.000 |
| 150.00 | 9.17 | 357.88 | 149.53 | 2.500 | 1.833 | 0.000 | 8.78 | 8.78 | 0.00 | 2.500 | 0.000 |
| 170.50 | 10.87 | 357.88 | 169.72 | 2.500 | 1.913 | 0.000 | 12.35 | 12.35 | 0.00 | 2.500 | 0.000 |
| 173.00 | 10.87 | 357.88 | 172.17 | 0.000 | 1.886 | 0.000 | 12.82 | 12.82 | 0.00 | 0.000 | 0.000 |
| 180.00 | 12.80 | 357.88 | 179.02 | 8.250 | 2.133 | 0.000 | 14.25 | 14.26 | 0.00 | 8.250 | 0.000 |
| 195.00 | 16.92 | 357.88 | 193.52 | 8.250 | 2.604 | 0.000 | 18.10 | 18.10 | 0.00 | 8.250 | 0.000 |
| 210.00 | 21.05 | 357.88 | 207.70 | 8.250 | 3.007 | 0.000 | 22.98 | 22.98 | 0.00 | 8.250 | 0.000 |
| 225.00 | 25.17 | 357.88 | 221.49 | 8.250 | 3.357 | 0.000 | 28.87 | 28.87 | 0.00 | 8.250 | 0.000 |
| 240.00 | 29.30 | 357.88 | 234.83 | 8.250 | 3.662 | 0.000 | 35.73 | 35.73 | 0.00 | 8.250 | 0.000 |
| 255.00 | 33.42 | 357.88 | 247.63 | 8.250 | 3.932 | 0.000 | 43.53 | 43.54 | 0.00 | 8.250 | 0.000 |
| 270.00 | 37.55 | 357.88 | 259.84 | 8.250 | 4.172 | 0.000 | 52.24 | 52.24 | 0.00 | 8.250 | 0.000 |
| 285.00 | 41.67 | 357.88 | 271.40 | 8.250 | 4.387 | 0.000 | 61.80 | 61.80 | 0.00 | 8.250 | 0.000 |
| 300.73 | 46.00 | 357.88 | 282.74 | 8.250 | 4.589 | 0.000 | 72.69 | 72.69 | 0.00 | 8.250 | 0.000 |
| 320.76 | 47.00 | 357.88 | 296.52 | 1.500 | 4.396 | 0.000 | 87.22 | 87.22 | 0.00 | 1.500 | 0.000 |
| 330.00 | 49.65 | 357.97 | 302.67 | 8.595 | 4.514 | 0.000 | 94.12 | 94.13 | 0.01 | 8.592 | 0.294 |
| 345.00 | 53.94 | 358.10 | 311.94 | 8.595 | 4.691 | 0.000 | 105.90 | 105.91 | 0.04 | 8.592 | 0.266 |
| 360.00 | 58.24 | 358.22 | 320.31 | 8.595 | 4.854 | 0.000 | 118.35 | 118.36 | 0.10 | 8.593 | 0.238 |
| 375.00 | 62.54 | 358.33 | 327.72 | 8.595 | 5.003 | 0.000 | 131.39 | 131.40 | 0.19 | 8.593 | 0.217 |
| 390.00 | 66.83 | 358.43 | 334.13 | 8.595 | 5.141 | 0.000 | 144.94 | 144.95 | 0.31 | 8.593 | 0.201 |
| 405.00 | 71.13 | 358.52 | 339.51 | 8.595 | 5.269 | 0.000 | 158.94 | 158.95 | 0.46 | 8.593 | 0.188 |
| 420.00 | 75.43 | 358.61 | 343.83 | 8.595 | 5.388 | 0.000 | 173.31 | 173.31 | 0.63 | 8.593 | 0.179 |
| 435.00 | 79.72 | 358.70 | 347.05 | 8.595 | 5.499 | 0.000 | 187.95 | 187.95 | 0.83 | 8.593 | 0.172 |
| 450.00 | 84.02 | 358.78 | 349.17 | 8.595 | 5.602 | 0.000 | 202.80 | 202.80 | 1.05 | 8.593 | 0.167 |
| 465.00 | 88.32 | 358.87 | 350.17 | 8.595 | 5.698 | 0.000 | 217.76 | 217.76 | 1.30 | 8.593 | 0.165 |
| 467.38 | 89.00 | 358.88 | 350.23 | 8.595 | 5.713 | 0.000 | 220.14 | 220.14 | 1.34 | 8.593 | 0.164 |
| 480.00 | 89.00 | 358.88 | 350.45 | 0.000 | 5.563 | 0.000 | 232.76 | 232.76 | 1.56 | 0.000 | 0.000 |
| 510.00 | 89.00 | 358.88 | 350.97 | 0.000 | 5.236 | 0.000 | 262.75 | 262.75 | 2.08 | 0.000 | 0.000 |
| 540.00 | 89.00 | 358.88 | 351.50 | 0.000 | 4.945 | 0.000 | 292.75 | 292.74 | 2.61 | 0.000 | 0.000 |
| 570.00 | 89.00 | 358.88 | 352.02 | 0.000 | 4.685 | 0.000 | 322.74 | 322.73 | 3.13 | 0.000 | 0.000 |
| 600.00 | 89.00 | 358.88 | 352.54 | 0.000 | 4.450 | 0.000 | 352.74 | 352.72 | 3.65 | 0.000 | 0.000 |
| 630.00 | 89.00 | 358.88 | 353.07 | 0.000 | 4.239 | 0.000 | 382.73 | 382.71 | 4.18 | 0.000 | 0.000 |
| 660.00 | 89.00 | 358.88 | 353.59 | 0.000 | 4.046 | 0.000 | 412.73 | 412.70 | 4.70 | 0.000 | 0.000 |
| 690.00 | 89.00 | 358.88 | 354.12 | 0.000 | 3.870 | 0.000 | 442.72 | 442.69 | 5.22 | 0.000 | 0.000 |
| 720.00 | 89.00 | 358.88 | 354.64 | 0.000 | 3.709 | 0.000 | 472.72 | 472.68 | 5.75 | 0.000 | 0.000 |
| 750.00 | 89.00 | 358.88 | 355.16 | 0.000 | 3.560 | 0.000 | 502.71 | 502.67 | 6.27 | 0.000 | 0.000 |
| 767.06 | 89.00 | 358.88 | 355.46 | 0.000 | 3.481 | 0.000 | 519.76 | 519.72 | 6.57 | 0.000 | 0.000 |

  - Geothermal Gradient Data

| Ambient Temperature | 26.000 °C | Mudline Temperature | 4.444 °C |
| --- | --- | --- | --- |
| Temperature @ Depth | 30.000 °C @ 355.85 m | Gradient | 1.12 °C/100m |

- Schematics

| Well: | 2737 | Wellbore: | 2737 | Case: | 2159 | String Name: | 215,9 мм |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Torque & Drag Setup Data

  - Settings

| Measured Depth of Bit | 404.00 m | Bending Stress Magnification | Yes |
| --- | --- | --- | --- |
| Block Weight | 1.00 tonne | Stiff String Analysis | No |
| Enable Sheave Friction Correction | No | Viscous Torque and Drag | No |
| Pump Rate | 28.000 L/sec | Contact Force Normalization Length | 9.50 m |
| Mechanical Efficiency (Single Sheave) | 97.00 | Lines Strung | 12 |
|  |  | Side Force | 0.00 kgf |
| Offset from Wellhead | m | Angle at Wellhead | ° |
| Buckling limit factor | 1 |  |  |

  - Run Parameters

| Start MD | 160.00 m | End MD | 417.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Normal Analysis Operational Parameters

| Drilling | WOB/Overpull (tonne) | Torque at Bit (kN-m) | Include Pump Rate |
| --- | --- | --- | --- |
| Rotating On Bottom | 5.00 | 4.1670 | NA |
| Slide Drilling | 2.00 | 0.0000 | NA |
| Backreaming | NA | NA | NA |
| Rotating Off Bottom |  |  | NA |
| Tripping | Speed (m/min) | RPM (rpm) | Include Pump Rate |
| Tripping In | 10.00 | 0 | NA |
| Tripping Out | 10.00 | 0 | NA |

  - Friction Factors

| Section Type | Coefficient of Friction |
| --- | --- |
| Casing | 0.25 |
| Open Hole | 0.36 |

  - String Fill Up

| Use String Fill Up (Tripping In only) | No | Period | m |
| --- | --- | --- | --- |

- Torque and Drag Results

  - Mechanical Limitations

| Overpull Margin during a Tripping Out operation | 80.72 tonne | using | 80.00% of yield |
| --- | --- | --- | --- |
| Minimum Weight on Bit to Sinusoidal Buckle during a rotating on bottom operation | 15.55 tonne | at | 0.00 m |
| Minimum Weight on Bit to Helical Buckle during a rotating on bottom operation | 17.52 tonne | at | 0.00 m |
| Pick-Up Drag | 2.67 tonne |  |  |
| Slack-Off Drag | 2.35 tonne |  |  |
| Block Rating (Hoisting System) | 225.00 tonne |  |  |
| Torque Rating (Rotating Equipment) | kN-m |  |  |

  - Load Summary

| Load Condition | Stress Failure |  |  | Buckling Limits |  |  | Torque Failure | Torque at the Rotary Table (kN-m) | Total Windup with Bit Torque (revs) | Total Windup without Bit Torque (revs) | Measured Weight (tonne) | Total Stretch (m) | Axial Stress = 0 |  | Neutral Point Distance from surface (m) | Neutral Point Distance from Bit (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fatigue | 90% Yield | 100% Yield | Sinusoidal | Helical | Lockup |  |  |  |  |  |  | Distance from Surface (m) | Distance from Bit (m) |  |  |
| Спуск |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 9.45 | 0.03 | 289.88 | 114.12 | 404.00 | 0.00 |
| Подъём |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 14.47 | 0.05 | 346.57 | 57.43 | 404.00 | 0.00 |
| Бурение ротором |  |  |  |  |  |  |  | 6.7496 | 0.4 | 0.1 | 6.80 | 0.01 | 178.72 | 225.28 | 207.89 | 196.11 |
| Бурение ГЗД |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 6.92 | 0.02 | 187.25 | 216.75 | 221.60 | 182.40 |
| Вращение над забоем |  |  |  |  |  |  |  | 1.8959 | 0.1 | 0.1 | 11.80 | 0.04 | 346.57 | 57.43 | 404.00 | 0.00 |

- Torque and Drag Plots

- Эффективное натяжение

- Вес на крюке

- Момент

- Мин. вес на долоте

- Hydraulics Setup Data

  - Calculation Engine

| Model Used | WellPlan |  |
| --- | --- | --- |

  - Cuttings Loading Calculation Option

| Rate of Penetration | 8.00 m/hr | Rotary Speed | 40 rpm |
| --- | --- | --- | --- |
| Cuttings Diameter | 3.18 mm | Cuttings Density | 2.500 sg |
| Bed Porosity | 36.00 % | MD Calculation Interval | 30.48 m |

  - Surface Equipment Information

| Pressure Loss Calculation | Specify Pressure loss | Maximum Working Pressure | 270.0000 atm |
| --- | --- | --- | --- |
| Equipment Mode | NA | Surface Pressure Loss | 6.8046 atm |
| Equipment Type | NA |  |  |

  - Pump Pressure Information

| Maximum Surface Pressure | 340.0000 atm | Pump Rate | 28.000 L/sec |
| --- | --- | --- | --- |
| Maximum Pump Power | hp | Maximum Allowable Pump Rate | L/sec |
| Use Roughness | N |  |  |
| Pipe Roughness | NA | Annulus Roughness | NA |
| Booster Pump |  | Injection Depth |  |
| Injection Temperature |  | Injection Rate |  |
| Include Tool Joint Pressure Losses | N |  |  |
| Include Back Pressure |  | Back Pressure | 0.0000 atm |
| Sea Floor Returns | N | Sea Water Density | NA |

  - Run Parameters

| Start MD | 160.00 m | End MD | 417.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Flow Rate (Q= 28.000 L/sec)

    - Bit Parameters

| Pump Rate | 28.000 L/sec | Stand Pipe Pressure | 94.7333 atm |
| --- | --- | --- | --- |
| Bit Pressure Loss | 21.3277 atm | Percent Power at Bit | 22.51 % |
| Bit Hydraulic Power / Area (HSI) | 1.4 hp/in² | Bit Nozzle Velocity | 57.74 m/s |
| Bit Hydraulic Power | 81.14 hp | Bit Impact Force | 192.89 kgf |
| Surface Equip. Pressure Loss | 6.8046 atm | Total Bit Flow Area | 0.752 in² |

  - Gel Strength

| 0 Second | 5.000 lbf/100ft² | 10 Second | 9.000 lbf/100ft² |
| --- | --- | --- | --- |
| 10 Minute | 18.000 lbf/100ft² | 30 Minute | 25.000 lbf/100ft² |
| Maximum | lbf/100ft² |  |  |

  - Mud Temperature Information

| Include Mud Temperature Effects | N | Circulation Time | NA |
| --- | --- | --- | --- |

- Hydraulics Plots

- Потери давления на долоте

- Мин. расход по глубине

- Высота шламовой подушки по глубине

- Потери мощности компонента

- Потери давления компонента