WellPlanTM Report

04. КМГ-Бурение

Well Name: 2758

Wellbore: 2758

Design: Copy Of Design #2 РК от 25.08.23 (1)

Case: Copy Of 152,4мм новый (1)

Date: October 5, 2023 at 11:16 AM

Created By:

- General Information

  - General Case Information

| Company | 04. КМГ-Бурение |  |  |
| --- | --- | --- | --- |
| Project | Молдабек | Site | 2758_Акт выноса |
| Well | 2758 | Wellbore | 2758 |
| Design | Copy Of Design #2 РК от 25.08.23 (1) | Case | Copy Of 152,4мм новый (1) |
| Hole MD | 828.42 m | Hole TVD | 263.59 m |
| Air Gap | 5.20 m | Ground Elevation | 84.32 m |
| Reference Point | WELL (copy) (copy) @ 89.52m | Well Type | Platform |

  - Active Fluid

    - Fluid Data

| Fluid | KC1 | Type | Mud |
| --- | --- | --- | --- |
| Mud Base Type | Water | Base Fluid | Water |
| Rheology Model | Bingham Plastic | Foamed |  |

    - Rheology Data

| Temperature (°C) | Pressure (atm) | Base Density (kg/m³) | Ref Fluid Properties | PV (Mulnf) (cp) | YP (Tau0) (lbf/100ft²) | Fann Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Speed (rpm) | Dial (°) |
| 30 | 1 | 1130 | Yes | 12 | 20 |  |  |

  - Hole Section

| Section Type | Section Depth (m) | Section Length (m) | Shoe Depth (m) | ID (mm) | Drift (mm) | Eff. Hole Diameter (mm) | Coefficient of Friction | Linear Capacity (L/m) | Volume Excess (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Casing | 433 | 433 | 433 | 161.7 | 158.75 |  | 0.25 | 20.54 |  |
| Open Hole | 828.42 | 395.42 |  |  |  | 152.4 | 0.3 | 18.24 | 0 |

  - String Details

| Type | Length (m) | Depth (m) | Body |  | Stabilizer / Tool Joint |  |  |  | Weight | Material | Grade | Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OD (mm) | ID (mm) | Avg Joint Length (m) | Length (m) | OD (mm) | ID (mm) |  |  |  |  |

| Drill Pipe | 9.225 | 9.23 | 101.6 | 82.3 | 9.144 | 0.433 | 137.32 | 76.2 | 26.41 | CS_API 5D/7 | G | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drill Collar | 160 | 169.23 | 120.65 | 50.8 | 9.144 |  |  |  | 73.83 | CS_API 5D/7 | 4145H MOD |  |
| Heavy Weight | 160 | 329.23 | 88.9 | 57.15 | 9.144 | 1.219 | 120.65 | 58.75 | 34.53 | CS_1340 MOD | 1340 MOD |  |
| Drill Pipe | 470 | 799.23 | 88.9 | 66.09 | 9.144 | 0.469 | 117.48 | 53.98 | 25.12 | CS_API 5D/7 | G | 2 |
| Sub | 0.91 | 800.14 | 120.65 | 28 | 0.91 |  |  |  | 64.63 | CS_API 5D/7 | 4145H MOD |  |
| MWD | 10.44 | 810.58 | 120.65 | 40.64 | 10.44 |  |  |  | 85.87 | SAE 4145 | SAE 4145 |  |
| MWD | 9.1 | 819.68 | 120.65 | 40.64 | 9.1 |  |  |  | 85.87 | SAE 4145 | SAE 4145 |  |
| Stabilizer | 1.524 | 821.2 | 104 | 31.75 | 1.524 |  |  |  | 43.75 | CS_API 5D/7 | 4145H MOD |  |
| Mud Motor | 7 | 828.2 | 120.65 | 63.5 | 7 |  |  |  | 57.37 | CS_API 5D/7 | 4145H MOD |  |
| Bit | 0.221 | 828.42 | 152.4 |  | 0.221 |  |  |  | 126.49 |  |  |  |

    - Grade in Use

| Grade | Minimum Yield Stress (psi) |
| --- | --- |
| 1340 MOD | 55,000 |
| 4145H MOD | 110,000 |
| G | 105,000 |
| SAE 4145 | 110,000 |

    - String Nozzles

| Component | MD (m) | Port Open | Diverted Flow | Amount Diverted (%) | Nozzle (32nd") | TFA (in²) |
| --- | --- | --- | --- | --- | --- | --- |
| Polycrystalline Diamond Bit | 828 | NA | NA | NA | 5.0X16.0 | 0.982 |

    - Mud Motors

| Component | Length (m) | Steering tool |  |  | Kick pad |  |  | Pressure loss @ Flow rate ( @ ) | Lobe config | Eccentricity () | Rotor mass () | Rev. per Volume () |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bend angle (°) | Ref angle (°) | Offset (m) | Length (m) | OD (mm) | Offset (m) |  |  |  |  |  |

|  |  |  |  |  |  |  |  | @ @ @ @ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Tortuosity (Random Inc and Az)

| Tortuosity Period | 100.00 m |
| --- | --- |
| Interpolation Interval | 9.14 m |

| Measured Depth Top (m) | Magnitude (°) |
| --- | --- |
| 0.00 | 0.50 |
| 100.00 | 1.50 |
| 200.00 | 1.50 |
| 300.00 | 1.50 |
| 400.00 | 1.50 |
| 500.00 | 1.00 |
| 600.00 | 1.00 |
| 700.00 | 1.00 |
| 828.42 | 1.00 |

  - Wellpath - Calculation Method: Minimum Curvature

| MD (m) | INC (°) | AZ (°) | TVD (m) | DLS (°/30m) | AbsTort (°/30m) | RelTort (°/30m) | VSect (m) | NS (m) | EW (m) | Build (°/30m) | Walk (°/30m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 | 357.89 | 0.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 30.00 | 0.00 | 357.89 | 30.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 52.00 | 0.00 | 357.89 | 52.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 60.00 | 2.39 | 346.87 | 60.00 | 8.966 | 1.195 | 0.000 | 0.17 | 0.16 | -0.03 | 8.966 | 0.000 |
| 75.00 | 6.87 | 346.87 | 74.94 | 8.966 | 2.750 | 0.000 | 1.37 | 1.35 | -0.26 | 8.966 | 0.000 |
| 90.00 | 11.36 | 346.87 | 89.75 | 8.966 | 3.786 | 0.000 | 3.73 | 3.68 | -0.72 | 8.966 | 0.000 |
| 105.00 | 15.84 | 346.87 | 104.33 | 8.966 | 4.526 | 0.000 | 7.22 | 7.15 | -1.39 | 8.966 | 0.000 |
| 120.00 | 20.32 | 346.87 | 118.58 | 8.966 | 5.081 | 0.000 | 11.84 | 11.71 | -2.28 | 8.966 | 0.000 |
| 135.00 | 24.81 | 346.87 | 132.43 | 8.966 | 5.512 | 0.000 | 17.56 | 17.36 | -3.38 | 8.966 | 0.000 |
| 150.00 | 29.29 | 346.87 | 145.79 | 8.966 | 5.858 | 0.000 | 24.32 | 24.06 | -4.69 | 8.966 | 0.000 |
| 165.00 | 33.77 | 346.87 | 158.57 | 8.966 | 6.140 | 0.000 | 32.11 | 31.75 | -6.18 | 8.966 | 0.000 |
| 180.00 | 38.25 | 346.87 | 170.70 | 8.966 | 6.376 | 0.000 | 40.86 | 40.41 | -7.87 | 8.966 | 0.000 |
| 195.00 | 42.74 | 346.87 | 182.10 | 8.966 | 6.575 | 0.000 | 50.52 | 49.97 | -9.73 | 8.966 | 0.000 |
| 210.00 | 47.22 | 346.87 | 192.71 | 8.966 | 6.746 | 0.000 | 61.04 | 60.37 | -11.76 | 8.966 | 0.000 |
| 225.00 | 51.70 | 346.87 | 202.46 | 8.966 | 6.894 | 0.000 | 72.36 | 71.56 | -13.94 | 8.966 | 0.000 |
| 240.00 | 56.19 | 346.87 | 211.28 | 8.966 | 7.023 | 0.000 | 84.39 | 83.46 | -16.26 | 8.966 | 0.000 |
| 255.00 | 60.67 | 346.87 | 219.14 | 8.966 | 7.138 | 0.000 | 97.07 | 96.00 | -18.70 | 8.966 | 0.000 |
| 270.00 | 65.15 | 346.87 | 225.96 | 8.966 | 7.239 | 0.000 | 110.32 | 109.10 | -21.25 | 8.966 | 0.000 |
| 285.00 | 69.64 | 346.87 | 231.73 | 8.966 | 7.330 | 0.000 | 124.06 | 122.69 | -23.90 | 8.966 | 0.000 |
| 300.00 | 74.12 | 346.87 | 236.39 | 8.966 | 7.412 | 0.000 | 138.21 | 136.68 | -26.62 | 8.966 | 0.000 |
| 311.68 | 77.61 | 346.87 | 239.25 | 8.966 | 7.470 | 0.000 | 149.45 | 147.80 | -28.79 | 8.966 | 0.000 |
| 328.69 | 77.61 | 346.87 | 242.90 | 0.000 | 7.084 | 0.000 | 165.94 | 164.11 | -31.97 | 0.000 | 0.000 |
| 330.00 | 77.98 | 346.87 | 243.17 | 8.436 | 7.089 | 0.000 | 167.20 | 165.36 | -32.21 | 8.436 | 0.069 |
| 345.00 | 82.20 | 346.90 | 245.75 | 8.436 | 7.147 | 0.000 | 181.87 | 179.86 | -35.03 | 8.436 | 0.067 |
| 360.00 | 86.41 | 346.94 | 247.24 | 8.436 | 7.201 | 0.000 | 196.68 | 194.51 | -37.87 | 8.436 | 0.066 |
| 365.70 | 88.02 | 346.95 | 247.52 | 8.436 | 7.220 | 0.000 | 202.33 | 200.10 | -38.95 | 8.435 | 0.066 |
| 390.00 | 88.01 | 349.38 | 248.36 | 3.000 | 6.957 | 0.000 | 226.49 | 224.04 | -43.05 | -0.007 | 3.002 |
| 420.00 | 88.01 | 352.38 | 249.40 | 3.000 | 6.675 | 0.000 | 256.43 | 253.79 | -46.71 | -0.002 | 3.002 |
| 450.00 | 88.01 | 355.38 | 250.45 | 3.000 | 6.430 | 0.000 | 286.41 | 283.70 | -48.80 | 0.003 | 3.002 |
| 480.04 | 88.02 | 358.39 | 251.49 | 3.000 | 6.215 | 0.000 | 316.38 | 313.70 | -49.33 | 0.009 | 3.002 |
| 510.00 | 88.02 | 358.39 | 252.52 | 0.000 | 5.850 | 0.000 | 346.24 | 343.65 | -49.07 | 0.000 | 0.000 |
| 540.00 | 88.02 | 358.39 | 253.56 | 0.000 | 5.525 | 0.000 | 376.13 | 373.63 | -48.81 | 0.000 | 0.000 |
| 570.00 | 88.02 | 358.39 | 254.59 | 0.000 | 5.234 | 0.000 | 406.02 | 403.61 | -48.54 | 0.000 | 0.000 |
| 600.00 | 88.02 | 358.39 | 255.63 | 0.000 | 4.973 | 0.000 | 435.91 | 433.59 | -48.28 | 0.000 | 0.000 |
| 630.00 | 88.02 | 358.39 | 256.67 | 0.000 | 4.736 | 0.000 | 465.79 | 463.57 | -48.02 | 0.000 | 0.000 |
| 660.00 | 88.02 | 358.39 | 257.70 | 0.000 | 4.520 | 0.000 | 495.68 | 493.56 | -47.76 | 0.000 | 0.000 |
| 690.00 | 88.02 | 358.39 | 258.74 | 0.000 | 4.324 | 0.000 | 525.57 | 523.54 | -47.50 | 0.000 | 0.000 |
| 720.00 | 88.02 | 358.39 | 259.78 | 0.000 | 4.144 | 0.000 | 555.46 | 553.52 | -47.24 | 0.000 | 0.000 |
| 750.00 | 88.02 | 358.39 | 260.81 | 0.000 | 3.978 | 0.000 | 585.35 | 583.50 | -46.97 | 0.000 | 0.000 |
| 780.00 | 88.02 | 358.39 | 261.85 | 0.000 | 3.825 | 0.000 | 615.24 | 613.48 | -46.71 | 0.000 | 0.000 |
| 810.00 | 88.02 | 358.39 | 262.89 | 0.000 | 3.683 | 0.000 | 645.13 | 643.46 | -46.45 | 0.000 | 0.000 |
| 828.32 | 88.02 | 358.39 | 263.52 | 0.000 | 3.602 | 0.000 | 663.38 | 661.77 | -46.29 | 0.000 | 0.000 |
| 828.42 | 88.02 | 358.39 | 263.52 | 0.000 | 3.601 | 0.000 | 663.48 | 661.87 | -46.29 | 0.000 | 0.000 |

  - Pore Pressure

| True Vertical Depth (TVD) (m) | Pore Pressure (atm) | Equivalent Mud Weight (EMW) (kg/m³) |
| --- | --- | --- |
| 5.20 | 0.5024 | 998 |

  - Fracture Gradient

| True Vertical Depth (TVD) (m) | Fracture Pressure (atm) | Equivalent Mud Weight (EMW) (kg/m³) |
| --- | --- | --- |
| 5.20 | 0.0000 | 0 |

  - Geothermal Gradient Data

| Ambient Temperature | 25.000 °C | Mudline Temperature | °C |
| --- | --- | --- | --- |
| Temperature @ Depth | 20.000 °C @ 263.59 m | Gradient | -1.94 °C/100m |

- Schematics

| Well: | 2758 | Wellbore: | 2758 | Case: | Copy Of 152,4мм новый (1) | String Name: | 152,4 |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Torque & Drag Setup Data

  - Settings

| Measured Depth of Bit | 828.42 m | Bending Stress Magnification | Yes |
| --- | --- | --- | --- |
| Block Weight | 17.00 tonne | Stiff String Analysis | No |
| Enable Sheave Friction Correction | No | Viscous Torque and Drag | No |
| Pump Rate | 15.000 L/sec | Contact Force Normalization Length | 9.45 m |
| Mechanical Efficiency (Single Sheave) | 97.00 | Lines Strung | 12 |
|  |  | Side Force | kgf |
| Offset from Wellhead | m | Angle at Wellhead | ° |
| Buckling limit factor | 1 |  |  |

  - Run Parameters

| Start MD | 0.00 m | End MD | 828.56 m |
| --- | --- | --- | --- |
| Step Size | 10.00 m |  |  |

  - Normal Analysis Operational Parameters

| Drilling | WOB/Overpull (tonne) | Torque at Bit (kN-m) | Include Pump Rate |
| --- | --- | --- | --- |
| Rotating On Bottom | 5.00 | 1.3730 | NA |
| Slide Drilling | 2.00 | 0.0000 | NA |
| Backreaming | NA | NA | NA |
| Rotating Off Bottom |  |  | NA |
| Tripping | Speed (m/min) | RPM (rpm) | Include Pump Rate |
| Tripping In | 10.00 | 0 | NA |
| Tripping Out | 10.00 | 0 | NA |

  - Friction Factors

| Section Type | Coefficient of Friction |
| --- | --- |
| Casing | 0.25 |
| Open Hole | 0.30 |

  - String Fill Up

| Use String Fill Up (Tripping In only) | No | Period | 0.00 m |
| --- | --- | --- | --- |

- Torque and Drag Results

  - Mechanical Limitations

| Overpull Margin during a Tripping Out operation | 65.31 tonne | using | 80.00% of yield |
| --- | --- | --- | --- |
| Minimum Weight on Bit to Sinusoidal Buckle during a rotating on bottom operation | 12.90 tonne | at | 18.11 m |
| Minimum Weight on Bit to Helical Buckle during a rotating on bottom operation | 13.28 tonne | at | 18.11 m |
| Pick-Up Drag | 4.75 tonne |  |  |
| Slack-Off Drag | 5.83 tonne |  |  |
| Block Rating (Hoisting System) | 250.00 tonne |  |  |
| Torque Rating (Rotating Equipment) | kN-m |  |  |

  - Load Summary

| Load Condition | Stress Failure |  |  | Buckling Limits |  |  | Torque Failure | Torque at the Rotary Table (kN-m) | Total Windup with Bit Torque (revs) | Total Windup without Bit Torque (revs) | Measured Weight (tonne) | Total Stretch (m) | Axial Stress = 0 |  | Neutral Point Distance from surface (m) | Neutral Point Distance from Bit (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fatigue | 90% Yield | 100% Yield | Sinusoidal | Helical | Lockup |  |  |  |  |  |  | Distance from Surface (m) | Distance from Bit (m) |  |  |
| Спуск |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 23.94 | -0.15 | 100.94 | 727.48 | 828.42 | 0.00 |
| Подъём |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 34.52 | -0.10 | 763.07 | 65.35 | 828.42 | 0.00 |
| Бурение ротором |  |  |  |  |  |  |  | 4.8720 | 0.9 | 0.5 | 24.77 | -0.18 | 111.27 | 717.15 | 130.78 | 697.64 |
| Бурение ГЗД |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 21.12 | -0.18 | 60.85 | 767.57 | 71.04 | 757.38 |
| Вращение над забоем |  |  |  |  |  |  |  | 2.7985 | 0.5 | 0.5 | 29.77 | -0.12 | 262.90 | 565.52 | 828.42 | 0.00 |

- Torque and Drag Plots

- Эффективное натяжение

- Вес на крюке

- Момент

- Мин. вес на долоте

- Hydraulics Setup Data

  - Calculation Engine

| Model Used | WellPlan |  |
| --- | --- | --- |

  - Cuttings Loading Calculation Option

| Rate of Penetration | 10.00 m/hr | Rotary Speed | 25 rpm |
| --- | --- | --- | --- |
| Cuttings Diameter | 3.18 mm | Cuttings Density | 2.145 sg |
| Bed Porosity | 36.00 % | MD Calculation Interval | 10.00 m |

  - Surface Equipment Information

| Pressure Loss Calculation | Specify Pressure loss | Maximum Working Pressure | atm |
| --- | --- | --- | --- |
| Equipment Mode | NA | Surface Pressure Loss | 0.0000 atm |
| Equipment Type | NA |  |  |

  - Pump Pressure Information

| Maximum Surface Pressure | 350.0000 atm | Pump Rate | 15.000 L/sec |
| --- | --- | --- | --- |
| Maximum Pump Power | hp | Maximum Allowable Pump Rate | L/sec |
| Use Roughness | N |  |  |
| Pipe Roughness | NA | Annulus Roughness | NA |
| Booster Pump |  | Injection Depth |  |
| Injection Temperature |  | Injection Rate |  |
| Include Tool Joint Pressure Losses |  |  |  |
| Include Back Pressure |  | Back Pressure | atm |
| Sea Floor Returns | N | Sea Water Density | NA |

  - Run Parameters

| Start MD | 0.00 m | End MD | 828.56 m |
| --- | --- | --- | --- |
| Step Size | 10.00 m |  |  |

  - Flow Rate (Q= 15.000 L/sec)

    - Bit Parameters

| Pump Rate | 15.000 L/sec | Stand Pipe Pressure | 117.0027 atm |
| --- | --- | --- | --- |
| Bit Pressure Loss | 3.4653 atm | Percent Power at Bit | 2.96 % |
| Bit Hydraulic Power / Area (HSI) | 0.2 hp/in² | Bit Nozzle Velocity | 23.68 m/s |
| Bit Hydraulic Power | 7.06 hp | Bit Impact Force | 40.93 kgf |
| Surface Equip. Pressure Loss | 0.0000 atm | Total Bit Flow Area | 0.982 in² |

  - Gel Strength

| 0 Second | lbf/100ft² | 10 Second | lbf/100ft² |
| --- | --- | --- | --- |
| 10 Minute | lbf/100ft² | 30 Minute | lbf/100ft² |
| Maximum | lbf/100ft² |  |  |

  - Mud Temperature Information

| Include Mud Temperature Effects | N | Circulation Time | NA |
| --- | --- | --- | --- |

- Hydraulics Plots

- Потери давления на долоте

- Мин. расход по глубине

- Высота шламовой подушки по глубине

- Потери мощности компонента

- Потери давления компонента