WellPlanTM Report

04. КМГ-Бурение

Well Name: Н1-ЮЗК

Wellbore: Н1-ЮЗК

Design: План №1 от 06.08.20

Case: 149.2мм

Date: October 5, 2023 at 9:45 PM

Created By:

- General Information

  - General Case Information

| Company | 04. КМГ-Бурение |  |  |
| --- | --- | --- | --- |
| Project | Ю.З. Камышитовый | Site | Н-1 с альтитудой |
| Well | Н1-ЮЗК | Wellbore | Н1-ЮЗК |
| Design | План №1 от 06.08.20 | Case | 149.2мм |
| Hole MD | 630.00 m | Hole TVD | 217.47 m |
| Air Gap | 0.00 m | Ground Elevation | -22.00 m |
| Reference Point | WELL (copy) (copy) (copy) @ -22m | Well Type | Platform |

  - Active Fluid

    - Fluid Data

| Fluid | KC1 | Type | Mud |
| --- | --- | --- | --- |
| Mud Base Type | Water | Base Fluid | Water |
| Rheology Model | Bingham Plastic | Foamed | N |

    - Rheology Data

| Temperature (°C) | Pressure (atm) | Base Density (kg/m³) | Ref Fluid Properties | PV (Mulnf) (cp) | YP (Tau0) (lbf/100ft²) | Fann Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Speed (rpm) | Dial (°) |
| 21 | 1 | 1120 | Yes | 16 | 22 |  |  |

  - Hole Section

| Section Type | Section Depth (m) | Section Length (m) | Shoe Depth (m) | ID (mm) | Drift (mm) | Eff. Hole Diameter (mm) | Coefficient of Friction | Linear Capacity (L/m) | Volume Excess (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Casing | 195 | 195 | 195 | 161.7 | 158.53 | 226.6 | 0.25 | 20.54 |  |
| Open Hole | 630 | 435 |  | 149.2 | 220.45 | 149.2 | 0.3 | 17.48 | 0 |

  - String Details

| Type | Length (m) | Depth (m) | Body |  | Stabilizer / Tool Joint |  |  |  | Weight | Material | Grade | Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | OD (mm) | ID (mm) | Avg Joint Length (m) | Length (m) | OD (mm) | ID (mm) |  |  |  |  |

| Drill Pipe | 100.609 | 100.61 | 101.6 | 82.3 | 9.144 | 0.433 | 127.4 | 61.91 | 26.12 | CS_API 5D/7 | G | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Heavy Weight | 110 | 210.61 | 88.9 | 57.15 | 9.144 | 1.219 | 120.65 | 58.75 | 34.53 | CS_1340 MOD | 1340 MOD |  |
| Jar | 10.058 | 220.67 | 120.65 | 50.8 | 10.058 |  |  |  | 73.84 | CS_API 5D/7 | 4145H MOD |  |
| Heavy Weight | 150 | 370.67 | 88.9 | 57.15 | 9.14 | 1.219 | 120.65 | 58.75 | 34.53 | CS_1340 MOD | 1340 MOD |  |
| Drill Pipe | 80 | 450.67 | 101.6 | 82.3 | 9.144 | 0.433 | 127.4 | 61.91 | 26.12 | CS_API 5D/7 | G | 2 |
| Drill Pipe | 150 | 600.67 | 88.9 | 66.09 | 9.144 | 0.469 | 117.48 | 53.98 | 25.12 | CS_API 5D/7 | G | 2 |
| MWD | 10.44 | 611.11 | 120.65 | 40.64 | 10.44 |  |  |  | 85.87 | SAE 4145 | SAE 4145 |  |
| MWD | 9.1 | 620.21 | 120.65 | 40.64 | 9.1 |  |  |  | 85.87 | SAE 4145 | SAE 4145 |  |
| Stabilizer | 1.524 | 621.73 | 120.65 | 44.45 | 1.524 |  |  |  | 43.75 | CS_API 5D/7 | 4145H MOD |  |
| Sub | 0.914 | 622.65 | 112.78 | 30.48 | 0.914 |  |  |  | 74.32 | CS_API 5D/7 | 4145H MOD |  |
| Mud Motor | 7 | 629.65 | 120.65 | 63.5 | 7 |  |  |  | 57.37 | CS_API 5D/7 | 4145H MOD |  |
| Bit | 0.355 | 630 | 149.2 |  | 0.355 |  |  |  | 0.5 |  |  |  |

    - Grade in Use

| Grade | Minimum Yield Stress (psi) |
| --- | --- |
| 1340 MOD | 55,000 |
| 4145H MOD | 110,000 |
| G | 105,000 |
| SAE 4145 | 110,000 |

    - String Nozzles

| Component | MD (m) | Port Open | Diverted Flow | Amount Diverted (%) | Nozzle (32nd") | TFA (in²) |
| --- | --- | --- | --- | --- | --- | --- |
| Polycrystalline Diamond Bit | 630 | NA | NA | NA | 2.0X14.0 | 0.632 |
| Polycrystalline Diamond Bit | 630 | NA | NA | NA | 3.0X12.0 | 0.632 |

    - Mud Motors

| Component | Length (m) | Steering tool |  |  | Kick pad |  |  | Pressure loss @ Flow rate ( @ ) | Lobe config | Eccentricity () | Rotor mass () | Rev. per Volume () |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bend angle (°) | Ref angle (°) | Offset (m) | Length (m) | OD (mm) | Offset (m) |  |  |  |  |  |

|  |  |  |  |  |  |  |  | @ @ @ @ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Tortuosity (Random Inc and Az)

| Tortuosity Period | 30.00 m |
| --- | --- |
| Interpolation Interval | 9.14 m |

| Measured Depth Top (m) | Magnitude (°) |
| --- | --- |
| 0.00 | 0.50 |
| 35.00 | 1.50 |

  - Wellpath - Calculation Method: Minimum Curvature

| MD (m) | INC (°) | AZ (°) | TVD (m) | DLS (°/30m) | AbsTort (°/30m) | RelTort (°/30m) | VSect (m) | NS (m) | EW (m) | Build (°/30m) | Walk (°/30m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 | 0.15 | 0.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 20.00 | 0.00 | 0.15 | 20.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 30.00 | 0.00 | 0.15 | 30.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 35.00 | 0.00 | 0.15 | 35.00 | 0.000 | 0.000 | 0.000 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 45.00 | 2.93 | 232.85 | 45.00 | 8.800 | 1.956 | 0.000 | 0.25 | -0.16 | -0.20 | 8.800 | 0.000 |
| 60.00 | 7.33 | 232.85 | 59.93 | 8.800 | 3.667 | 0.000 | 1.59 | -0.97 | -1.27 | 8.800 | 0.000 |
| 75.00 | 11.73 | 232.85 | 74.72 | 8.800 | 4.693 | 0.000 | 4.05 | -2.47 | -3.25 | 8.800 | 0.000 |
| 90.00 | 16.13 | 232.85 | 89.28 | 8.800 | 5.378 | 0.000 | 7.64 | -4.66 | -6.12 | 8.800 | 0.000 |
| 105.00 | 20.53 | 232.85 | 103.51 | 8.800 | 5.867 | 0.000 | 12.32 | -7.52 | -9.87 | 8.800 | 0.000 |
| 120.00 | 24.93 | 232.85 | 117.34 | 8.800 | 6.233 | 0.000 | 18.08 | -11.03 | -14.48 | 8.800 | 0.000 |
| 135.00 | 29.33 | 232.85 | 130.69 | 8.800 | 6.519 | 0.000 | 24.87 | -15.18 | -19.92 | 8.800 | 0.000 |
| 150.00 | 33.73 | 232.85 | 143.47 | 8.800 | 6.747 | 0.000 | 32.65 | -19.93 | -26.16 | 8.800 | 0.000 |
| 165.00 | 38.13 | 232.85 | 155.61 | 8.800 | 6.933 | 0.000 | 41.39 | -25.26 | -33.16 | 8.800 | 0.000 |
| 180.00 | 42.53 | 232.85 | 167.04 | 8.800 | 7.089 | 0.000 | 51.03 | -31.14 | -40.88 | 8.800 | 0.000 |
| 195.50 | 47.08 | 232.85 | 178.04 | 8.800 | 7.225 | 0.000 | 61.87 | -37.76 | -49.57 | 8.800 | 0.000 |
| 200.00 | 48.84 | 232.85 | 181.05 | 11.715 | 7.326 | 0.000 | 65.19 | -39.79 | -52.23 | 11.715 | 0.000 |
| 210.00 | 52.74 | 232.85 | 187.37 | 11.715 | 7.535 | 0.000 | 72.88 | -44.48 | -58.39 | 11.715 | 0.000 |
| 220.00 | 56.65 | 232.85 | 193.15 | 11.715 | 7.725 | 0.000 | 80.99 | -49.43 | -64.88 | 11.715 | 0.000 |
| 230.00 | 60.55 | 232.85 | 198.36 | 11.715 | 7.898 | 0.000 | 89.46 | -54.60 | -71.67 | 11.715 | 0.000 |
| 240.00 | 64.46 | 232.85 | 202.98 | 11.715 | 8.057 | 0.000 | 98.27 | -59.97 | -78.72 | 11.715 | 0.000 |
| 250.00 | 68.36 | 232.85 | 206.98 | 11.715 | 8.203 | 0.000 | 107.36 | -65.52 | -86.01 | 11.715 | 0.000 |
| 260.00 | 72.27 | 232.85 | 210.34 | 11.715 | 8.339 | 0.000 | 116.71 | -71.23 | -93.50 | 11.715 | 0.000 |
| 270.00 | 76.17 | 232.85 | 213.06 | 11.715 | 8.464 | 0.000 | 126.26 | -77.06 | -101.15 | 11.715 | 0.000 |
| 280.00 | 80.08 | 232.85 | 215.12 | 11.715 | 8.580 | 0.000 | 135.98 | -82.99 | -108.94 | 11.715 | 0.000 |
| 290.00 | 83.98 | 232.85 | 216.51 | 11.715 | 8.688 | 0.000 | 145.81 | -88.99 | -116.81 | 11.715 | 0.000 |
| 300.00 | 87.89 | 232.85 | 217.22 | 11.715 | 8.789 | 0.000 | 155.71 | -95.03 | -124.75 | 11.715 | 0.000 |
| 305.41 | 90.00 | 232.85 | 217.32 | 11.715 | 8.841 | 0.000 | 161.08 | -98.31 | -129.05 | 11.715 | 0.000 |
| 330.00 | 90.00 | 230.39 | 217.32 | 3.000 | 8.405 | 0.000 | 185.56 | -113.63 | -148.28 | 0.000 | -3.000 |
| 360.00 | 90.00 | 227.39 | 217.32 | 3.000 | 7.955 | 0.000 | 215.51 | -133.41 | -170.83 | 0.000 | -3.000 |
| 390.00 | 90.00 | 224.39 | 217.32 | 3.000 | 7.574 | 0.000 | 245.51 | -154.34 | -192.32 | 0.000 | -3.000 |
| 420.00 | 90.00 | 221.39 | 217.32 | 3.000 | 7.247 | 0.000 | 275.46 | -176.37 | -212.67 | 0.000 | -3.000 |
| 432.41 | 90.00 | 220.15 | 217.32 | 3.000 | 7.125 | 0.000 | 287.82 | -185.79 | -220.75 | 0.000 | -3.000 |
| 450.00 | 90.00 | 220.15 | 217.32 | 0.000 | 6.847 | 0.000 | 305.32 | -199.27 | -232.06 | 0.000 | 0.000 |
| 480.00 | 90.00 | 220.15 | 217.32 | 0.000 | 6.419 | 0.000 | 335.16 | -222.25 | -251.34 | 0.000 | 0.000 |
| 510.00 | 90.00 | 220.15 | 217.32 | 0.000 | 6.041 | 0.000 | 365.00 | -245.23 | -270.63 | 0.000 | 0.000 |
| 540.00 | 90.00 | 220.15 | 217.32 | 0.000 | 5.706 | 0.000 | 394.84 | -268.21 | -289.91 | 0.000 | 0.000 |
| 570.00 | 90.00 | 220.15 | 217.32 | 0.000 | 5.405 | 0.000 | 424.68 | -291.19 | -309.20 | 0.000 | 0.000 |
| 600.00 | 90.00 | 220.15 | 217.32 | 0.000 | 5.135 | 0.000 | 454.53 | -314.17 | -328.48 | 0.000 | 0.000 |
| 630.00 | 90.00 | 220.15 | 217.32 | 0.000 | 4.890 | 0.000 | 484.37 | -337.15 | -347.76 | 0.000 | 0.000 |

  - Geothermal Gradient Data

| Ambient Temperature | 26.000 °C | Mudline Temperature | °C |
| --- | --- | --- | --- |
| Temperature @ Depth | 21.000 °C @ 217.47 m | Gradient | -2.30 °C/100m |

- Schematics

| Well: | Н1-ЮЗК | Wellbore: | Н1-ЮЗК | Case: | 149.2мм | String Name: | 149.2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Torque & Drag Setup Data

  - Settings

| Measured Depth of Bit | 630.00 m | Bending Stress Magnification | Yes |
| --- | --- | --- | --- |
| Block Weight | 17.00 tonne | Stiff String Analysis | No |
| Enable Sheave Friction Correction | No | Viscous Torque and Drag | No |
| Pump Rate | 16.000 L/sec | Contact Force Normalization Length | 9.30 m |
| Mechanical Efficiency (Single Sheave) | 97.00 | Lines Strung | 12 |
|  |  | Side Force | 0.00 kgf |
| Offset from Wellhead | m | Angle at Wellhead | ° |
| Buckling limit factor | 1 |  |  |

  - Run Parameters

| Start MD | 195.00 m | End MD | 630.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Normal Analysis Operational Parameters

| Drilling | WOB/Overpull (tonne) | Torque at Bit (kN-m) | Include Pump Rate |
| --- | --- | --- | --- |
| Rotating On Bottom | 4.00 | 1.3730 | NA |
| Slide Drilling | 1.00 | 0.0000 | NA |
| Backreaming | NA | NA | NA |
| Rotating Off Bottom |  |  | NA |
| Tripping | Speed (m/min) | RPM (rpm) | Include Pump Rate |
| Tripping In | 10.00 | 0 | NA |
| Tripping Out | 10.00 | 0 | NA |

  - Friction Factors

| Section Type | Coefficient of Friction |
| --- | --- |
| Casing | 0.25 |
| Open Hole | 0.30 |

  - String Fill Up

| Use String Fill Up (Tripping In only) | No | Period | m |
| --- | --- | --- | --- |

- Torque and Drag Results

  - Mechanical Limitations

| Overpull Margin during a Tripping Out operation | 45.60 tonne | using | 80.00% of yield |
| --- | --- | --- | --- |
| Minimum Weight on Bit to Sinusoidal Buckle during a rotating on bottom operation | 6.63 tonne | at | 25.15 m |
| Minimum Weight on Bit to Helical Buckle during a rotating on bottom operation | 7.15 tonne | at | 25.15 m |
| Pick-Up Drag | 3.90 tonne |  |  |
| Slack-Off Drag | 5.13 tonne |  |  |
| Block Rating (Hoisting System) | 225.00 tonne |  |  |
| Torque Rating (Rotating Equipment) | kN-m |  |  |

  - Load Summary

| Load Condition | Stress Failure |  |  | Buckling Limits |  |  | Torque Failure | Torque at the Rotary Table (kN-m) | Total Windup with Bit Torque (revs) | Total Windup without Bit Torque (revs) | Measured Weight (tonne) | Total Stretch (m) | Axial Stress = 0 |  | Neutral Point Distance from surface (m) | Neutral Point Distance from Bit (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fatigue | 90% Yield | 100% Yield | Sinusoidal | Helical | Lockup |  |  |  |  |  |  | Distance from Surface (m) | Distance from Bit (m) |  |  |
| Спуск |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 17.82 | 0.23 | 31.39 | 598.61 | 630.00 | 0.00 |
| Подъём |  |  |  |  |  |  |  | 0.0000 | 0.0 | 0.0 | 26.85 | 0.28 | 575.67 | 54.33 | 630.00 | 0.00 |
| Бурение ротором |  |  |  |  |  |  |  | 4.4168 | 0.7 | 0.4 | 18.95 | 0.22 | 74.80 | 555.20 | 86.44 | 543.56 |
| Бурение ГЗД |  |  |  | X |  |  |  | 0.0000 | 0.0 | 0.0 | 16.26 | 0.22 | 0.00 | 630.00 | 0.00 | 630.00 |
| Вращение над забоем |  |  |  |  |  |  |  | 2.3978 | 0.3 | 0.3 | 22.95 | 0.25 | 210.61 | 419.39 | 630.00 | 0.00 |

- Torque and Drag Plots

- Эффективное натяжение

- Вес на крюке

- Момент

- Мин. вес на долоте

- Hydraulics Setup Data

  - Calculation Engine

| Model Used | WellPlan |  |
| --- | --- | --- |

  - Cuttings Loading Calculation Option

| Rate of Penetration | 8.00 m/hr | Rotary Speed | 80 rpm |
| --- | --- | --- | --- |
| Cuttings Diameter | 3.18 mm | Cuttings Density | 2.500 sg |
| Bed Porosity | 36.00 % | MD Calculation Interval | 30.48 m |

  - Surface Equipment Information

| Pressure Loss Calculation | Specify Pressure loss | Maximum Working Pressure | 270.0000 atm |
| --- | --- | --- | --- |
| Equipment Mode | NA | Surface Pressure Loss | 6.8046 atm |
| Equipment Type | NA |  |  |

  - Pump Pressure Information

| Maximum Surface Pressure | 340.0000 atm | Pump Rate | 16.000 L/sec |
| --- | --- | --- | --- |
| Maximum Pump Power | hp | Maximum Allowable Pump Rate | L/sec |
| Use Roughness | N |  |  |
| Pipe Roughness | NA | Annulus Roughness | NA |
| Booster Pump |  | Injection Depth |  |
| Injection Temperature |  | Injection Rate |  |
| Include Tool Joint Pressure Losses | N |  |  |
| Include Back Pressure |  | Back Pressure | 0.0000 atm |
| Sea Floor Returns | N | Sea Water Density | NA |

  - Run Parameters

| Start MD | 195.00 m | End MD | 630.00 m |
| --- | --- | --- | --- |
| Step Size | 9.30 m |  |  |

  - Flow Rate (Q= 16.000 L/sec)

    - Bit Parameters

| Pump Rate | 16.000 L/sec | Stand Pipe Pressure | 142.8757 atm |
| --- | --- | --- | --- |
| Bit Pressure Loss | 9.4297 atm | Percent Power at Bit | 6.60 % |
| Bit Hydraulic Power / Area (HSI) | 0.8 hp/in² | Bit Nozzle Velocity | 39.24 m/s |
| Bit Hydraulic Power | 20.50 hp | Bit Impact Force | 71.71 kgf |
| Surface Equip. Pressure Loss | 6.8046 atm | Total Bit Flow Area | 0.632 in² |

  - Gel Strength

| 0 Second | 5.000 lbf/100ft² | 10 Second | 9.000 lbf/100ft² |
| --- | --- | --- | --- |
| 10 Minute | 18.000 lbf/100ft² | 30 Minute | 25.000 lbf/100ft² |
| Maximum | lbf/100ft² |  |  |

  - Mud Temperature Information

| Include Mud Temperature Effects | N | Circulation Time | NA |
| --- | --- | --- | --- |

- Hydraulics Plots

- Потери давления на долоте

- Мин. расход по глубине

- Высота шламовой подушки по глубине

- Потери мощности компонента

- Потери давления компонента